/*

**

**                           Main.c

**

**

**********************************************************************/

/*

Last committed:     $Revision: 00 $

Last changed by:    $Author: Ryšavý Jindřich$

Last changed date:  $Date:  $

ID:                 $Id:  $

**********************************************************************/

#include "stm32f4xx_conf.h"

//other user includes

#include "arm_math.h"

//user defines

//DAC signals

#define WF_DEPTH 15

#define DAC_PORT GPIOC

//buttons must be  at GPIO 10 - GPIO 15, each button different number, port does not matter

#define BTN_UP_PORT     GPIOB

#define BTN_UP          GPIO_Pin_15

#define BTN_DWN_PORT    GPIOB

#define BTN_DWN         GPIO_Pin_14

#define BTN_LEFT_PORT   GPIOB

#define BTN_LEFT        GPIO_Pin_13

#define BTN_RIGHT_PORT  GPIOB

#define BTN_RIGHT       GPIO_Pin_12

#define BTN_START_PORT  GPIOB

#define BTN_START       GPIO_Pin_10

#define MENU_NO 3

//variables

volatile static uint32_t btn_state;

static float tenth_of_hz_step;

volatile static uint32_t gen_step;

static uint16_t wf[1<<(WF_DEPTH)];

//function prototypes

void startup_config (void);

//void EXTI15_10_IRQHandler(void);  //just for indication where to find it

void settings_update (void);

void lcd_update (void);

uint32_t dds_gen(volatile uint32_t *step_ptr, uint32_t init_phase) __attribute__ ((hot));

int main(void)

{

startup_config();

uint32_t phase = 0;

while (1)

{

settings_update();

lcd_update();

if (gen_step && !btn_state)

{

NVIC_InitTypeDef NVIC_InitStruct;

NVIC_InitStruct.NVIC_IRQChannel = EXTI15_10_IRQn;

NVIC_InitStruct.NVIC_IRQChannelPreemptionPriority = 0x02;

NVIC_InitStruct.NVIC_IRQChannelSubPriority = 0x03;

NVIC_InitStruct.NVIC_IRQChannelCmd = ENABLE;

EXTI_ClearITPendingBit(BTN_UP);

EXTI_ClearITPendingBit(BTN_DWN);

EXTI_ClearITPendingBit(BTN_LEFT);

EXTI_ClearITPendingBit(BTN_RIGHT);

EXTI_ClearITPendingBit(BTN_START);

asm volatile("":::"memory");

NVIC_Init(&NVIC_InitStruct);

phase = dds_gen(&gen_step,phase);

}

}

}

void startup_config (void)

{

//Clocks

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOA, ENABLE);

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOB, ENABLE);

RCC_AHB1PeriphClockCmd(RCC_AHB1Periph_GPIOC, ENABLE);

RCC_APB2PeriphClockCmd(RCC_APB2Periph_SYSCFG, ENABLE);

//Initialize pins

GPIO_InitTypeDef GPIO_InitDef;

GPIO_InitDef.GPIO_Pin = BTN_UP;         //buttons

GPIO_InitDef.GPIO_OType = GPIO_OType_PP;

GPIO_InitDef.GPIO_Mode = GPIO_Mode_IN;

GPIO_InitDef.GPIO_PuPd = GPIO_PuPd_UP;

GPIO_InitDef.GPIO_Speed = GPIO_Speed_2MHz;

GPIO_Init(BTN_UP_PORT, &GPIO_InitDef);

GPIO_InitDef.GPIO_Pin = BTN_DWN;

GPIO_Init(BTN_DWN_PORT, &GPIO_InitDef);

GPIO_InitDef.GPIO_Pin = BTN_RIGHT;

GPIO_Init(BTN_RIGHT_PORT, &GPIO_InitDef);

GPIO_InitDef.GPIO_Pin = BTN_LEFT;

GPIO_Init(BTN_LEFT_PORT, &GPIO_InitDef);

GPIO_InitDef.GPIO_Pin = BTN_START;

GPIO_Init(BTN_START_PORT, &GPIO_InitDef);

GPIO_InitDef.GPIO_Pin = GPIO_Pin_All;   //DAC sample

GPIO_InitDef.GPIO_OType = GPIO_OType_PP;

GPIO_InitDef.GPIO_Mode = GPIO_Mode_OUT;

GPIO_InitDef.GPIO_PuPd = GPIO_PuPd_NOPULL;

GPIO_InitDef.GPIO_Speed = GPIO_Speed_25MHz;

GPIO_Init(DAC_PORT, &GPIO_InitDef);

GPIO_InitDef.GPIO_Pin = GPIO_Pin_5;     //nucleo f401 user led, feel free to change if needed

GPIO_InitDef.GPIO_OType = GPIO_OType_PP;

GPIO_InitDef.GPIO_Mode = GPIO_Mode_OUT;

GPIO_InitDef.GPIO_PuPd = GPIO_PuPd_NOPULL;

GPIO_InitDef.GPIO_Speed = GPIO_Speed_50MHz;

GPIO_Init(GPIOA, &GPIO_InitDef);

//SystemCoreClock

SystemCoreClockUpdate();

float size_of_acc = (float) (1<<31) *-2;

uint32_t ten_sample_freq = SystemCoreClock*(10/5); //dds loop does sample per 5 clocks

tenth_of_hz_step = size_of_acc / (float)(ten_sample_freq);

//buttons interrupt

SYSCFG_EXTILineConfig(EXTI_PortSourceGPIOB, EXTI_PinSource15);

SYSCFG_EXTILineConfig(EXTI_PortSourceGPIOB, EXTI_PinSource14);

SYSCFG_EXTILineConfig(EXTI_PortSourceGPIOB, EXTI_PinSource13);

SYSCFG_EXTILineConfig(EXTI_PortSourceGPIOB, EXTI_PinSource12);

SYSCFG_EXTILineConfig(EXTI_PortSourceGPIOB, EXTI_PinSource10);

EXTI_InitTypeDef EXTI_InitStruct;

EXTI_InitStruct.EXTI_Line = BTN_UP|BTN_DWN|BTN_RIGHT|BTN_LEFT|BTN_START;

EXTI_InitStruct.EXTI_Mode = EXTI_Mode_Interrupt;

EXTI_InitStruct.EXTI_Trigger = EXTI_Trigger_Falling;

EXTI_InitStruct.EXTI_LineCmd = ENABLE;

EXTI_Init(&EXTI_InitStruct);

NVIC_PriorityGroupConfig(NVIC_PriorityGroup_2);

/*NVIC_InitTypeDef NVIC_InitStruct;

NVIC_InitStruct.NVIC_IRQChannel = EXTI15_10_IRQn;

NVIC_InitStruct.NVIC_IRQChannelPreemptionPriority = 0x02;

NVIC_InitStruct.NVIC_IRQChannelSubPriority = 0x03;

NVIC_InitStruct.NVIC_IRQChannelCmd = ENABLE;

NVIC_Init(&NVIC_InitStruct);

*/

}

void EXTI15_10_IRQHandler(void)

{

btn_state = 0;

gen_step = 0;

if (EXTI_GetITStatus(BTN_UP) != RESET)

btn_state |= BTN_UP;

if (EXTI_GetITStatus(BTN_DWN) != RESET)

btn_state |= BTN_DWN;

if (EXTI_GetITStatus(BTN_LEFT) != RESET)

btn_state |= BTN_LEFT;

if (EXTI_GetITStatus(BTN_RIGHT) != RESET)

btn_state |= BTN_RIGHT;

/*if (EXTI_GetITStatus(BTN_START) != RESET)

btn_state |= BTN_START;

*/

NVIC_InitTypeDef NVIC_InitStruct;

NVIC_InitStruct.NVIC_IRQChannel = EXTI15_10_IRQn;

NVIC_InitStruct.NVIC_IRQChannelPreemptionPriority = 0x02;

NVIC_InitStruct.NVIC_IRQChannelSubPriority = 0x03;

NVIC_InitStruct.NVIC_IRQChannelCmd = DISABLE;

NVIC_Init(&NVIC_InitStruct);

}

void settings_update (void)

{

static uint32_t ten_freq = 10000;   //default 1kHz

static uint32_t ten_freq_step = 1000;

typedef enum {Sinus, Square, Step}menu_items;

static menu_items menu = Sinus;     //default Sinus

switch(btn_state)

{

case BTN_UP:

{

if (menu==0)

menu=MENU_NO-1;

else

menu--;

while(!GPIO_ReadInputDataBit(BTN_UP_PORT, BTN_UP));

}

break;

case BTN_DWN:

{

if (menu<(MENU_NO-1))

menu++;

else

menu=0;

while(!GPIO_ReadInputDataBit(BTN_DWN_PORT, BTN_DWN));

}

break;

case BTN_LEFT:

{

if(menu==Step)

{

if(ten_freq_step==10000000)

ten_freq_step=1;

else

ten_freq_step *= 10;

while(!GPIO_ReadInputDataBit(BTN_LEFT_PORT, BTN_LEFT));

}

if ((menu==Sinus)||(menu==Square))

{

ten_freq+=ten_freq_step;

if (ten_freq>=SystemCoreClock/(5*2)) //sampling theorem

ten_freq=SystemCoreClock/(5*2);

while(!GPIO_ReadInputDataBit(BTN_LEFT_PORT, BTN_LEFT));

}

}

break;

case BTN_RIGHT:

{

if(menu==Step)

{

if(ten_freq_step==1)

ten_freq_step=10000000;

else

ten_freq_step /= 10;

while(!GPIO_ReadInputDataBit(BTN_RIGHT_PORT, BTN_RIGHT));

}

if ((menu==Sinus)||(menu==Square))

{

if (ten_freq>=ten_freq_step)

ten_freq-=ten_freq_step;

while(!GPIO_ReadInputDataBit(BTN_RIGHT_PORT, BTN_RIGHT));

}

}

break;

case BTN_START://gen start

{

uint32_t i;

if(menu==Step)

break;

//gen wf

if(menu==Sinus)

{   for(i=0;(i<(1<<(WF_DEPTH)));i++)

{

float sin_value = sinf( ((float)i *2*PI) / (float)(1<<(WF_DEPTH)) );

wf[i]=(0xFFFF/2)*( 1 + sin_value );

}

}

if(menu==Square)

{

for(i=0;(i<(1<<(WF_DEPTH)));i++)

{

if(i<(1<<(WF_DEPTH-1)))

wf[i]=0xFFFF;

else

wf[i]=0x0;

}

//for extra something on edges

wf[0]=0x7FFF;

wf[(1<<(WF_DEPTH-1))]=0x7FFF;

}

gen_step = (float)ten_freq * (float)tenth_of_hz_step;

while(!GPIO_ReadInputDataBit(BTN_START_PORT, BTN_START));

}

break;

default:

break;

}

//update buttons

/*

btn_state = GPIO_ReadInputDataBit(BTN_UP_PORT, BTN_UP); //redo, func returns 1/0

btn_state |= GPIO_ReadInputDataBit(BTN_DWN_PORT, BTN_DWN);

btn_state |= GPIO_ReadInputDataBit(BTN_LEFT_PORT, BTN_LEFT);

btn_state |= GPIO_ReadInputDataBit(BTN_RIGHT_PORT, BTN_RIGHT);

btn_state |= GPIO_ReadInputDataBit(BTN_START_PORT, BTN_START);

*/

btn_state = 0;

if (!GPIO_ReadInputDataBit(BTN_UP_PORT, BTN_UP))

btn_state |= BTN_UP;

if (!GPIO_ReadInputDataBit(BTN_DWN_PORT, BTN_DWN))

btn_state |= BTN_DWN;

if (!GPIO_ReadInputDataBit(BTN_LEFT_PORT, BTN_LEFT))

btn_state |= BTN_LEFT;

if (!GPIO_ReadInputDataBit(BTN_RIGHT_PORT, BTN_RIGHT))

btn_state |= BTN_RIGHT;

if (!GPIO_ReadInputDataBit(BTN_START_PORT, BTN_START))

btn_state |= BTN_START;

}

void lcd_update (void)

{

// empty for now

}

uint32_t dds_gen(volatile uint32_t *step_ptr, uint32_t init_phase)  //returns phase of next (not generated) sample

{

register uint32_t acc1;

register uint32_t acc2;

register uint32_t acc3;

register uint32_t acc4;

register uint32_t step;

register uint32_t wf_offset1;

register uint16_t wf_sample1;

register uint32_t wf_offset2;

register uint16_t wf_sample2;

register uint32_t wf_offset3;

register uint16_t wf_sample3;

register uint32_t wf_offset4;

register uint16_t wf_sample4;

//most of these will be optimized out, still needed to describe exact instruction sequence

acc1 = init_phase;

step = *step_ptr;

acc2 = acc1 + step;

wf_offset1 = (acc1>>(32-WF_DEPTH));

acc3 = acc2 + (step);

wf_offset2 = (acc2>>(32-WF_DEPTH));

acc4 = acc3 + (step);

wf_sample1 = wf[wf_offset1];

//reset timer for correct DAC latch sync

//not implemented yet

while (step)

{                                       //action    taken clocks

acc1 = acc4 + (step);               //1 add         0   //ALU before memory operation not messing with its address/data is free

asm volatile("":::"memory");

GPIO_Write(DAC_PORT, wf_sample1);   //1 write       2

asm volatile("":::"memory");

acc2 = acc1 + step;                 //2 add         1

wf_offset3 = (acc3>>(32-WF_DEPTH)); //3 shift       1

acc3 = acc2 + (step);               //3 add         0

asm volatile("":::"memory");

wf_sample2 = wf[wf_offset2];        //2 read        2

GPIO_Write(DAC_PORT, wf_sample2);   //2 write       1

asm volatile("":::"memory");

wf_offset4 = (acc4>>(32-WF_DEPTH)); //4 shift       1

acc4 = acc3 + (step);               //4 add         0

asm volatile("":::"memory");

step = *step_ptr;                   //step read     2

wf_sample3 = wf[wf_offset3];        //3 read        1

GPIO_Write(DAC_PORT, wf_sample3);   //3 write       1

asm volatile("":::"memory");

wf_offset1 = (acc1>>(32-WF_DEPTH)); //1 shift       0

wf_offset2 = (acc2>>(32-WF_DEPTH)); //2 shift       1

asm volatile("":::"memory");

wf_sample4 = wf[wf_offset4];        //4 read        2

wf_sample1 = wf[wf_offset1];        //1 read        1

GPIO_Write(DAC_PORT, wf_sample4);   //4 write       1

//  compare     1

//  branch      2

/* assembly equivalent(addresses, registers & constants may vary):

adds	r0, r2, r6

str.w	lr, [r1, #20]

add.w	lr, r0, r2

mov.w	r8, r5, lsr #29

add.w	r5, lr, r2

ldrh.w	r7, [r3, r7, lsl #1]

str	r7, [r1, #20]

mov.w	r12, r6, lsr #29

adds	r6, r5, r2

ldrh.w	r7, [r3, r8, lsl #1]

ldr	r2, [r4, #0]

str	r7, [r1, #20]

mov.w	r7, lr, lsr #29

mov.w	lr, r0, lsr #29

ldrh.w	r0, [r3, r12, lsl #1]

ldrh.w	lr, [r3, lr, lsl #1]

str	r0, [r1, #20]

cmp	r2, #0

bne.n	0x80003fa <main+174>

*/

}

return acc1;

}